🎓 Case Study: Understanding and Implementing the CPU Pipeline in Computer System Architecture

1. Introduction (1–2 minutes)

Modern CPUs are designed for speed and efficiency. One of the key architectural techniques that enable this is instruction pipelining — allowing multiple instructions to overlap during execution.
In this video, we’ll explore how CPU pipelining works, the problems it solves, and how pipeline hazards can be detected and minimized.

Purpose & Significance:

- Improves instruction throughput.

- Forms the foundation for superscalar and modern multicore processors.

- Essential for understanding how processors execute instructions efficiently.

2. Step-by-Step Process (8–10 minutes)

Step 1: The Concept of Pipelining

- Compare to an assembly line: each stage works on a part of an instruction.

- Basic 5-stage pipeline:

  - IF – Instruction Fetch

  - ID – Instruction Decode

  - EX – Execute

  - MEM – Memory Access

  - WB – Write Back

Step 2: Visualizing the Pipeline

- Show a diagram of the 5 stages with arrows showing flow.

- Example instruction sequence:

  - ADD R1, R2, R3

  - SUB R4, R1, R5

  - AND R6, R4, R7

- Display pipeline timing chart (show overlapping stages).

Step 3: Understanding Pipeline Hazards

- Structural Hazards: hardware resource conflict (e.g., single memory for both instructions and data).

- Data Hazards: instruction depends on a previous one’s result (RAW, WAR, WAW).

- Control Hazards: caused by branches or jumps.

Step 4: Techniques to Handle Hazards

- Forwarding (Bypassing): reuse result before it’s written back.

- Stalling: pause the pipeline until data is ready.

- Branch Prediction: guess the branch outcome to avoid pipeline flush.

Step 5: Simulation / Demo

- Use a simple animation or visual table showing pipeline progress.

- Demonstrate how stalling or forwarding resolves conflicts.

3. Visual Aids (throughout the video)

- Flowchart showing the 5 pipeline stages.

- Timing diagram (Gantt-style chart).

- Animated example showing instruction movement.

- Short simulation using PowerPoint or draw.io diagrams.

4. Summary (1–2 minutes)

To recap:

- CPU pipelining increases instruction throughput by overlapping stages.

- Hazards can reduce performance but are managed using techniques like forwarding and prediction.

- Understanding pipelining is essential for optimizing CPU performance and designing efficient processors.

Key takeaway:
Pipelining demonstrates how architecture transforms simple instruction execution into a highly parallel and optimized process.

5. Technical Quality Tips

- Record with clear microphone audio.

- Use screen recording (e.g., PowerPoint + voice-over).

- Ensure bright lighting if you appear on camera.

- Highlight key terms and arrows for animation transitions.

🎯 Rubric Alignment

| Criteria | Description | Points |
| --- | --- | --- |
| Content Depth | Explains CPU pipelining, stages, hazards, and solutions clearly. | 30 |
| Organization & Clarity | Logical flow: intro → concept → demo → summary. | 20 |
| Engagement & Presentation | Visuals, animation, and voice narration keep attention. | 15 |