MEMOREY MODEL:
“RICS ( Reduced Instruction Set Computer )”
1. Load–Store Architecture 
* RISC CPUs follow a load/store model: 
   - Only load and store instructions access memory.
   - All arithmetic and logic operations happen only on registers. 
Real-Life Example: 
“Think of a chef cooking on a kitchen counter.” 
• Ingredients (data) must first be taken from the fridge (memory) and placed on the counter (registers). 
• The chef does all cutting, mixing, and cooking on the counter (registers only). 
• Finished dishes are put back in the fridge (store). 

2. Harvard-like Memory System 
* Many RISC processors adopt a Harvard architecture (or modified Harvard):
    - Separate instruction and data memory paths.
    - Allows CPU to fetch instructions and load/store data at the same time.
 * This improves throughput and avoids the Von Neumann bottleneck (where code and data share one memory bus). 
Real-Life Example: 
 “Imagine you’re watching YouTube on your phone”
  -  The phone fetches the video instructions (the "script") at the same time it fetches video        frames (data).
  -  If both used the same single path, you’d get buffering (Von Neumann bottleneck). 

3. Memory Consistency Model
 * In modern RISC ISAs (like ARM, RISC-V, MIPS), memory models define ordering rules in multi-core or multi-threaded systems. 
* RISC-V memory model: 
  - Weak ordering by default (operations may be reordered for performance). 
  - Uses fence instructions to enforce ordering when needed. 
* ARM memory model: 
  - Also relaxed, with explicit barriers (DMB, DSB) to control ordering. 
Real-Life Example:
• Imagine two chefs (CPU cores) working in the same kitchen (shared memory). 
• If one chef updates the salt jar (writes to memory), the other chef may not immediately see the update unless there’s a rule/barrier (fence instruction) that forces synchronization.

“CISC (Complex Instruction Set Computer) Memory Model”

- Memory-to-Memory Operations – CPU can directly read/write and process data from memory (no strict need for load/store).

- Von Neumann Architecture – One memory space for both instructions and data (can cause bottleneck).

- Strong Memory Ordering – Operations usually appear in the order written, making programming simpler.

- Complex Instructions – One instruction can do multiple memory accesses at once.

“Real-Life Example”
Like a chef cooking directly inside the fridge:

- He cuts, mixes, and Cooks food without taking it out first.

- Fewer steps are needed, but it’s slower and more complicated, since the fridge isn’t meant for cooking.

Similarities of RISC and CISC:
1. Both use cache – To speed up memory access, modern RISC and CISC CPUs both depend on cache.
2. Both support high-level languages – Whether RISC or CISC, they must compile and run languages like C, Java, or Python.
3. Both Aim to execute programs efficiently – The goal is still speed, performance, and reliability.
4. Both use memory + registers – Even if used differently, both rely on registers for temporary storage and memory for long-term storage.
5. Both are CPU design philosophies – They define how processors handle instructions and memory.

Resources:
https://en.wikipedia.org/wiki/Load%E2%80%93store_architecture

https://en.wikipedia.org/wiki/Reduced_instruction_set_computer

https://projectf.io/posts/riscv-load-store/

https://en.wikipedia.org/wiki/LatticeMico32

https://arxiv.org/abs/1707.05923